|  | Pulse / Pulse Next 2 users | Pulse / Pulse Next 10 users | Pulse PRO | DigiTech | MiniTech | TechMaster |
| --- | --- | --- | --- | --- | --- | --- |
| Power Supply | 9V Battery Large Battery Box 12V power supply | 9V Battery Large Battery Box 12V power supply | 9V Battery Large Battery Box 12V power supply | 9V Battery Large Battery Box 12V power supply | 9V Battery Large Battery Box 12V power supply | 9V Battery Large Battery Box 12V power supply |
| Users | - 1 Manager - 1 User | - 1 Manager - 8 Users | - Up to 2 Masters - Up to 2 Managers - Up to 49 Users | - 1 Manager - Up to 28 Users | - 1 Master - 1 Manager - Up to 48 Users | - Up to 2 Masters - Up to 2 Managers - 1 Override Manager - 1 Administrator - 89 Users |
| Override user |  | 1 User* | Up to 10 Users | 1 User* | Up to 10 Users* | Up to 80 of the users, available in groups* |
| Time delay | 1-99 minutes | 1-99 minutes | 1-99 minutes | 1-99 minutes | 1-99 minutes | 1-99 minutes Variable Time Delay* |
| Dual mode* |  | Available | Available | Available | Available | Available (in groups) |
| Dallas keys* |  | Available | Available | Available | Available | Available |
| Audit* |  | 100 events w/o time and date | - Up to 8.000 events with time&date - Automatic DST | - Up to 8.000 events with time&date - Audit display - Automatic DST | - Up to 11.000 events with time&date - Audit display - Automatic DST | - Up to 15.000 events with time&date - Audit display - Automatic DST |
| Interface* |  | - Silent alarm - Remote disable opening | - Silent alarm - Remote disable opening - Remote time delay override | - Silent alarm - Remote disable opening - Remote time delay override | - Silent alarm - Remote disable opening - Remote time delay override | - Silent alarm - Remote disable functions - Remote management (TCP/IP, RS485) |
| Time lock |  |  |  |  | - Weekly opening and closing - 60 between closing and opening periods - Immediate & temporary time lock - Time programmed from the keypad or a PC | - 3 users tables, each with weekly opening and closing - 60 between closing and opening periods - Immediate & temporary time lock - Time programmed from the keypad or a PC - Opening extensions |
| Advanced features* | - Backlit keypad (Pulse Next) - Slim version (Pulse Next) | - Backlit keypad (Pulse Next) - Slim version (Pulse Next) | - Battery level test - Fully configurable by keypad (more than 80 configurations) - Codes expiry* - Door open alarm* - Bolt status output* - ERC – Emergency Rescue Code* |  | - Dynamic Codes - Multiple locks: 2 | - Dynamic codes - Codes expiry - Multiple locks: 15 |
| Certifications |  |  |  |  |  |  |

All locks systems have penalty and low battery warning

*Optional features available with additional hardware and/or as a programming request when purchased